الامتحان النهائي لمادة تصميم المنطق الرقمي (نموذج  أ)-الفصل الأول 2010-2011

الاسم (باللغة العربية):.......................................................

الشعبة:     أحد-ثلاثاء-خميس  ،             اثنين-أربعاء          الرقم في الشعبة:......

| 1 | a | b | c | d | e |  | 11 | a | b | c | d | e |  | 21 | a | b | c | d | e |  | 31 | a | b | c | d | e |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | a | b | c | d | e |  | 12 | a | b | c | d | e |  | 22 | a | b | c | d | e |  | 32 | a | b | c | d | e |
| 3 | a | b | c | d | e |  | 13 | a | b | c | d | e |  | 23 | a | b | c | d | e |  | 33 | a | b | c | d | e |
| 4 | a | b | c | d | e |  | 14 | a | b | c | d | e |  | 24 | a | b | c | d | e |  | 34 | a | b | c | d | e |
| 5 | a | b | c | d | e |  | 15 | a | b | c | d | e |  | 25 | a | b | c | d | e |  | 35 | a | b | c | d | e |
| 6 | a | b | c | d | e |  | 16 | a | b | c | d | e |  | 26 | a | b | c | d | e |  | 36 | a | b | c | d | e |
| 7 | a | b | c | d | e |  | 17 | a | b | c | d | e |  | 27 | a | b | c | d | e |  | 37 | a | b | c | d | e |
| 8 | a | b | c | d | e |  | 18 | a | b | c | d | e |  | 28 | a | b | c | d | e |  | 38 | a | b | c | d | e |
| 9 | a | b | c | d | e |  | 19 | a | b | c | d | e |  | 29 | a | b | c | d | e |  | 39 | a | b | c | d | e |
| 10 | a | b | c | d | e |  | 20 | a | b | c | d | e |  | 30 | a | b | c | d | e |  | 40 | a | b | c | d | e |

________________________________________________________________________

1.  How many address lines and  data lines are needed in a 256 K x 64 RAM:

- 18 address lines and 64 data lines

- 64 address lines and 18 data lines

- 218 address lines and 64 data lines

- 256  address lines and 18 data

- Non of the above

________________________________________________________________________

2. Simplifying the Boolean expression A’ B ( D’ + C’ D ) + B ( A + A’ C D )

would result in:

- A          b)     B             c)     D         d)    A+B         e)   Non of the given answers

________________________________________________________________________

3. How many BCD counters are needed to build a counter for the range 0-999:

- 1 BCD counter

- 3 BCD counters

- 9 BCD counters

- 999 BCD counters

- Non of the above

________________________________________________________________________

4. The decimal number 20 is expressed in Binary Coded Decimal (BCD) as:

a)      1010

b)     00100000

c)     10101

d)     10100

e)  Non of the above

________________________________________________________________________

5. The unsigned number (AF.16C) 16 is equivalent to:

- Non of the answers below is right

- (257.0554) 8

- (157.0558) 8

- (157.0568) 8

- (157.0534) 8

________________________________________________________________________

6. How many 256K x 8 RAMs are needed to build a 1 M x 8 RAM:

- 2                 b)    4                c)       8                       d)     16              e)    32

________________________________________________________________________

7. The size of an Encoder with n output lines is:

- 2n  x 1       b)   2n x n       c)     n2  x 1         d)    n x 2n     e) Non of the given answers

________________________________________________________________________

8. The content of a 4-bit register is initially 1101. The register is shifted six times to the

right with the serial input being 101101. What is the contents of the register after the

sixth shift :

a)   1110        b) 1011        c)   1101        d)  0111           e)    Non of the given answers

________________________________________________________________________

9.   Which of the following statements is true?

a)  Outputs of sequential circuits do not depend on previous events.

- Outputs of combinational circuits do not depend on previous events.

- Sequential logic devices don't contain state storage information.

- Sequential logic devices’ outputs are dependent on current inputs only.

- Non of the given statements is true

________________________________________________________________________

  - Which of these devices are sequential?

  - Exclusive-Or gate

  - Counter

  - Decoder

  - Multiplexer

  - Tri-State Buffer

________________________________________________________________________

- How many binary bits are necessary to represent 748 different numbers?

a)   7          b)   8         c)  9       d)  10     e)  Non of the given answers

___________________________________________________________________________

- A logic function F(A,B,C,D) was simplified without considering the don’t care conditions and the expression for F was: F = B' C' + AC'  D + A' B'  C + A' B D . However, when the don’t care conditions were considered the same function F is  reduced to: F = A' +  C' .   The don’t care minterms that are used in the simplifications are:

a) d ( A, B, C, D) = ∑ (10, 11, 14, 15)

b) d ( A, B, C, D) = ∑ (1,6,7,14,15)

c) d ( A, B, C, D) = ∑ (4, 6, 12)

d) d ( A, B, C, D) = ∑ (0,2,3,4,5,9,12,13)

e) Non of the above

___________________________________________________________________________

- The 7’s complement for (6235)7 is:

a) 	 (0431) 7   b)     (1542)7    	c)     (1543)7 	 d)       (0432)7    e) Non of the given answers

_____________________________________________________________

- A function 	F ( A, B, C, D, E ) =  BC' D , then the expression for the function F as sum of minterms is:

a)  F ( A, B, C, D, E ) = ∑ (5, 13, 21, 29)

b) F ( A, B, C, D, E ) = ∑ (10,11,12,13)

c)   F ( A, B, C, D, E ) = ∑ (10,11, 26, 27)

d)  F ( A, B, C, D, E ) = ∑ (10,11)

e) Non of the above

___________________________________________________________________________

- The number of flip flops that will be complemented in a 10 bit count up binary counter to reach the next count after the count 1110010111:

a) 	 10            b)    2   	            c)     4 	                 d)    3                 e) 5

________________________________________________________________________

- Given the numbers (1000100) 2 , (1000003) 8 , (1000002) 10 , (1000001) 16 :

a)   (1000100) 2  is the smallest

b)  (1000002) 10 is the biggest

c)  (1000100) 2 , (1000003) 8 , and (1000001) 16 have equal value

- Two of the given numbers are equal

- Non of the above is true

________________________________________________________________________

- The shown circuit can be rebuilt using a minimum of:

a)  4 NAND gates

b)   5 NAND gates

c)  4 NAND   and 1 NOR gates

d)  3 NAND gates

e)  Non of the above

_______________________________________________

- To implement the Boolean function F = A B  C we can use a 3-to-8 decoder and connect:

a)  6 different lines from its output to an OR gate

b)  5 different lines from its output to an OR gate

c)  3 different lines from its output to an OR gate

d)  4 different lines from its output to an OR gate

e)  7 different lines from its output to an OR gate

__________________________________________________________________

- The correct state diagram for the circuit shown is:

__________________________________________________________________

- Two ‘T’ Flip-Flops, A and B, are used to implement the shown state diagram. To go from state “AB = 01” to “AB = 11” we need:

a)  TA = 1, TB  = 0

b)  TA = 0, TB  = 1

c)  TA = 0, TB  = 0

d)  TA = 1, TB  = 1

e)  TA = 1, TB  = don’t care

_____________________________________________________________

  - A Universal Shift Register, USR, is connected as shown. S1=1, S0  =1 select the load operation. Initially Q3Q2Q1Q0  = 1010. After 2 clock cycles:

CLK

a)  Q3Q2Q1Q0  = 0000

b)  Q3Q2Q1Q0  = 1010

c)  Q3Q2Q1Q0  = 0101

d)  Q3Q2Q1Q0  = 1001

e)  Q3Q2Q1Q0  = 1111

______________________________________________

- A ripple counter is a counter that:

a)  works asynchronous

b)   works synchronous

c)  can be designed to work synchronously or asynchronously

d)  counts “up” synchronously and “down” asynchronously (or vise versa)

e)   counts “Binary” synchronously and “BCD” asynchronously

________________________________________________________________

- Which of these statements are true?

a) A minterm equation is an unreduced sum-of-products Boolean expression.

b) A minterm equation requires the least number of logic gates to implement

c) A maxterm equation is an unreduced sum-of-products Boolean expression.

d) A maxterm equation requires the least number of logic gates to implement

e) Non of the above

______________________________________

- The binary value of  S4 S3 S2 S1 S0

at the output of the circuit shown is:

- 10001

- 10000

- 10110

- 01001

- Non of the above

_______________________________________________________________

- The function F(x, y, z) implemented using the

decoder shown next can be expressed as

(decoder outputs are active low as shown):

- Σ (1, 3, 6)

- Σ (0, 2, 4, 5, 7)

- Π  (1, 3, 6)

- Π  (0, 2, 4, 5, 7)

- Non of the above

_____________________________________________________

- The minimum number of flip flops needed to build a counter that counts in the following sequence    8 → 10 → 12 → 14 → 8 → …..      is:

a)  2        b)  3         c)   4            d) 5                   e) non of the given answers __________________________________________________________

Simplifying the shown K-map to minimum

number of variables  would result in:

- X1Y1’ + X0Y1’Y0’ + X1X0Y0’

- X1Y1 + X0Y1Y0 + X1X0Y0

- X1’Y1 + X0’Y1Y0 + X1’X0’Y0

- X1’Y1’ + X0’Y1’Y0’ + X1’X0’Y0

e)        Non of the above

__________________________________________________________________

- Its desired to design a 4-bit shift register with parallel load capability. The register is to have the following four modes of operation:  parallel load, shift right, shift left,  no change. What are the necessary components needed to design the register:

- four  D flip-flops only

- four D flip-flops and one 4x1 multiplexer

- four D flip flops and four 4x1 multiplexers

- one D flip-flop and four 4x1 multiplexers

- one D flip-flop and one 4x1 multiplexer

____________________________________________________________________

- The  size of the ROM that will accommodate the truth table for a BCD-to-Seven-segment display decoder is :

- 4 x 7 ROM

- 4 x 16 ROM

- 16x 7  ROM

- 16 x 4 ROM

- 16 x 128 ROM

____________________________________________________________________

- Referring to the characteristic table of the J K flip flop, the characteristic equation for the complement output of the J K flip-flop can be expresses as :

- Q' (t +1)   =  J  Q  + K Q

- Q ' (t +1)   =  J  Q ' + K Q

- Q '(t +1)    =  J ' Q  + K Q'

- Q ' (t +1)   =  J  Q  + K ' Q '

- Q ' (t +1)   =  J'  Q'  + K  Q

_____________________________________________________________________

- A ROM implements the following four Boolean functions: A(x,y,z) = ∑(2,5,6),  B(x,y,z) = ∑(0,3,4), C(x,y,z) = ∑(4,6,7),  and D(x,y,z) = ∑(0,1,2). Considering the ROM as a memory, specify the memory contents at address '5': (101)b.

- 0000

- 1000

- 1111

- 0101

- Non of the above

________________________________________________________________________

- Consider the four outputs F3 F2 F1 F0, (where E

is the input to the two DeMultiplexers)

- It is impossible to have any output = 1 at any time.

- Only one output can be 1 at any time.

- Only three outputs maximum can be 1 at any time

- Only two outputs maximum can be 1 at any time.

- All four outputs can be 1 at any time.

____________________________________________________

- The diagram shown next represents

- PROM

- FPGA

- PLA

- PAL

- Non of the above

___________________________________________________________

- The value (–53) 10 represented using 8 bits in 2’s complement system would be equivalent to:

- (2EB) 16    b)   (FB) 16    c)    (2B) 16     d)   (CB) 16          e)   Non of the above

________________________________________________________________________

- Regarding bits, there are

- 16 in a byte

- 2 in a nibble

- 4 in a byte

- 4 in a nibble

- None of the above

________________________________________________________________________

- Which one of the following statements is wrong:

- x Å 0 = x

- x Å 1 = x

- x Å x = 0

- x Å x’ = 1

- x’ Å x’ = 0

_________________________________________________

- Referring to the figure which uses tri-state buffers,

which of the following cases makes F = 1,

- A = 0, B = 1, C = 0

- A = 0, B = 0, C = 1

- A = 1, B = 0, C = 0

- A = 0, B = 0, C = 0

- A = 1, B = 0, C = 1

________________________________________________

- A 4  16  decoder can be constructed using only:

- Two 3  8 line decoders with “Enable” and one AND gate.

- Two 3  8 line decoders with “Enable” and one OR gate.

- Four 2  4 line decoders with “Enable”.

- Four 2  4 line decoders with “Enable” and one Not gate

- Five 2x4 decoders with "Enable"

____________________________________________________________________

- To add 3 operands X + Y + Z, each one is 4 bits wide,

we need a minimum of:

8 half adders

X3X2X1X0

+ Y3Y2Y1Y0

+ Z3 Z2 Z1Z0

- 12 half adders

- 4 full adders and 4 half adders

- 8 full adders

- 8 full adders and 1 half adder.

_______________________________________________________________________

- Which of the following inputs are used with ROMs:

- input and output data lines

- read/write

- clock input

- all of the above are used

- non of the above are used

________________________________________________________________________

Key for form (a) of Logic final for the Fall of 2010-2011

| 1 | a | b | c | d | e |  | 11 | a | b | c | d | e |  | 21 | a | b | c | d | e |  | 31 | a | b | c | d | e |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | a | b | c | d | e |  | 12 | a | b | c | d | e |  | 22 | a | b | c | d | e |  | 32 | a | b | c | d | e |
| 3 | a | b | c | d | e |  | 13 | a | b | c | d | e |  | 23 | a | b | c | d | e |  | 33 | a | b | c | d | e |
| 4 | a | b | c | d | e |  | 14 | a | b | c | d | e |  | 24 | a | b | c | d | e |  | 34 | a | b | c | d | e |
| 5 | a | b | c | d | e |  | 15 | a | b | c | d | e |  | 25 | a | b | c | d | e |  | 35 | a | b | c | d | e |
| 6 | a | b | c | d | e |  | 16 | a | b | c | d | e |  | 26 | a | b | c | d | e |  | 36 | a | b | c | d | e |
| 7 | a | b | c | d | e |  | 17 | a | b | c | d | e |  | 27 | a | b | c | d | e |  | 37 | a | b | c | d | e |
| 8 | a | b | c | d | e |  | 18 | a | b | c | d | e |  | 28 | a | b | c | d | e |  | 38 | a | b | c | d | e |
| 9 | a | b | c | d | e |  | 19 | a | b | c | d | e |  | 29 | a | b | c | d | e |  | 39 | a | b | c | d | e |
| 10 | a | b | c | d | e |  | 20 | a | b | c | d | e |  | 30 | a | b | c | d | e |  | 40 | a | b | c | d | e |

________________________________________________________________________